[image: image1.jpg]GOOD WILL INSTRUMENT CO.,, LTD.

NO. 7-1, Jhongsing Road, Tucheng Dist., New Taipei City, 236, Taiwan

GE lnerK T (886) 22268-0389 F (886)2 2268-0639  www.gwinstek.com






GSP-730 Specifications

The specifications apply when the GSP-730 is powered on for at least 30 minutes under +20°C~+30°C.
	Frequency

	Frequency Range

	 
	Setting Range
	150kHz to 3GHz

	Center Frequency

	 
	Setting Resolution
	0.1MHz

	
	Accuracy
	within ±50kHz (frequency span : 0.3GHz to 2.6GHz, 20 ±5°C)

	Frequency Span

	 
	Setting range
	1MHz to 3GHz

	
	Accuracy
	within ±3%  (frequency span : 0.3GHz to 2.6GHz, 20 ±5°C)

	Resolution Bandwidth

	 
	Setting Range
	30KHz, 100KHz, 300KHz,1MHz,

	SSB Phase Noise

	 
	-85dBc / Hz (typical, 500kHz offset,  RBW : 30kHz, Sweep time: 1.5s, Span:1MHz@1GHz)

	Inherent Spurious Response

	 
	less than -45dBc @ -40dBm Ref. Level (typical less than -50dBc)


	Amplitude

	Reference Level

	 
	Input Range
	+20 to -40dBm

	
	Unit
	dBm, dBV, dBµV

	Average Noise Level

	 
	≤ -100dBm (typical, center frequency : 1GHz RBW : 30kHz)

	Frequency Characteristic

	 
	Accuracy : Within ±2dB (1GHz);SPAN:5MHz; Ref. Level 0dBm, input signal -10dBm;

	 
	within ±3.0dB @300MHz~2.6GHz, 
within ±6.0dB @ 80~300MHz, 2.6~3GHz

	Input

	 
	Input Impedance
	50ohm

	 
	Input VSWR
	less than 2.0@input att ≧10dB

	 
	Input damage level
	+30dBm (CW average power), 25VDC

	 
	Input connector
	N connector


	Sweep

	Sweep Time

	 
	Setting Range
	300ms to 8.4s, auto (not adjustable)

	 
	Accuracy
	within ±2% ( frequency span : full span)


	General

	Communication

	 
	Display
	640*480 RGB color LCD

	Interface

	 
	RS-232C
	Sub-D female-D 9 pins

	 
	USB Connector
	USB Host/Device full speed supported

	VGA Output

	 
	 
	Sub-D female 15 pins

	Power Source

	 
	 
	AC 100~240V, 50/60Hz


	Other

	 
	Operating Temperature
	5 to 45°C (Guaranteed at 25 ±5°C, without soft carrying case)

	 
	Operating Humidity
	Less than 45°C / 90%RH

	 
	Storage Temperature
	-20 to 60°C, less than 60°C / 70%RH

	 
	Dimensions
	296 (L) × 153 (W) × 105 (H) mm

	 
	Weight
	Approx. 2.2kg


